Huairou Solar Observatory Station

1.INTRODUCTION

Huairou Solar Observing Station (HSOS) is located on an small island near the north bank of Huairou Reservoir, 60 kms north of the downtown of Beijing. The water surrounded the station extend about 2 kms to the east, south and west of the observing tower. The observing dome is on the top of a 31-meter-high tower. The HSOS's telescope is characterized by its movable dome, which can be shifted away from the telescope when the seeing is favorable. The surrounding water reduces the temperature fluctuation at the telescope height, which is favorable for high resolution solar observing. The annual observation days are about 300.

- Latitude: 40.4N

- Longitude: 116.6E

- Altitude: 62m

- Summer Temperature: Mean 26° C , Extreme Maximum >40° C

- Winter Temperature: Mean -4° C, Extreme Minimum -20° C

- Precipitation: 500mm

- Nearest Cities: Huairou 10kms, Beijing 60kms

- TELESCOPE INSTRUMENT

- Name

- 10cm Full-Disc Vector Magnetograph

Objective

Diameter 100mm

Focal length 770.86mm

Field Of View

Full-disc 32' x 32'

Spectral lines

FeI 5324.19Å for photosphere

Birefringent Filter

Bandwidth 1/8Å

tunable range (-0.5Å ~ +0.5Å )

CCD

Size: 992 x 1004 pixels

Pixel size 2" x 2"

Temporal resolution

10 min

Spatial resolution

Better than 5"

Sensitivity

5     gauss for longitudinal magnetic field

120 gauss for transverse magnetic field

- 3.Data

- The raw data format got by the 10cm Full-Disc Vector Magnetograph Telescope is FIT, and the corresponding JPG picture is created by the related program from FIT. Two kinds of datas are tranformed to the directory meanwhile. However, as the problem of telescope ,the observation is interrupted in the year of 2009. Now the telescope is keeping in repair.